ILC3 IPCore Architecture v0.1

Version: 0.1
Date: 2025-12-01
Scope: This document summarizes the ILC3_0c_guard-phase based IPCore architecture, its basic I/O interface, RTL channel model presets, and link-budget level results extracted from the Q18/Q19/Q20 Python simulations.

1. System Overview

The ILC3_0c_guard-phase (ILC3_0c_gp) scheme is a 2-bit (4-level) line code intended as a drop-in alternative to conventional PAM4 on short-reach serial links (e.g. memory PHY / chip-to-chip). The key benefit is an SNR gain of roughly 3 dB at the same pre-FEC BER, which can be translated into either lower transmit power or higher link margin.

2. Link-Budget Summary (from Q18/Q19/Q20)

Q18/Q19/Q20 simulations sweep SNR over 4–20 dB for three 5-tap ISI channels (a/b/c) with FFE-only equalization (length 11, ridge 0.001). For each SNR point the pre-FEC BER was measured for PAM4 and ILC3_0c_gp, and then passed through a block FEC model (n = 256, k = 228, t = 15, R ≈ 0.8906) to estimate post-FEC BER.

Key numerical trends:

Pre-FEC region (BER ≈ 1e-2 to 1e-3): ILC3_0c_gp requires about 2.8–3.0 dB less SNR than PAM4 across channels a/b/c.

Post-FEC target BER ≈ 1e-12: the ~3 dB SNR advantage is preserved when both schemes are followed by the same (256,228,t=15) block FEC.

Interpretation: At a fixed link quality target, ILC3_0c_gp can achieve comparable reliability at roughly half the transmit power compared to PAM4, or equivalently provide about 3 dB extra link margin at the same power.

Example SNR requirements (representative, from Q19/Q20):

| Channel | Target | Scheme | SNR_req [dB] | ΔSNR vs PAM4 [dB] (ILC3 negative) |
| --- | --- | --- | --- | --- |
| a | pre-FEC BER ≈ 1e-3 | PAM4 | ≈ 16.9 |  |
| a | pre-FEC BER ≈ 1e-3 | ILC3_0c_gp | ≈ 14.2 | ≈ -2.7 |
| b | pre-FEC BER ≈ 1e-3 | PAM4 | ≈ 17.7 |  |
| b | pre-FEC BER ≈ 1e-3 | ILC3_0c_gp | ≈ 14.4 | ≈ -3.3 |
| c | pre-FEC BER ≈ 1e-3 | PAM4 | ≈ 17.0 |  |
| c | pre-FEC BER ≈ 1e-3 | ILC3_0c_gp | ≈ 14.3 | ≈ -2.7 |
| a/b/c | post-FEC BER ≈ 1e-12 | PAM4 | ≈ 15.5–16.0 |  |
| a/b/c | post-FEC BER ≈ 1e-12 | ILC3_0c_gp | ≈ 12.0 | ≈ -3.5 |

3. IPCore Top-Level I/O

Top module: ilc3_ipcore_top

| Signal | Dir/Width | Description |
| --- | --- | --- |
| clk | input | System clock. |
| rst_n | input | Active-low synchronous reset. |
| tx_sym[1:0] | input | 2-bit TX symbol (0..3) from controller/PHY MAC side. |
| tx_valid | input | TX symbol valid. |
| tx_ready | output | Handshake: core ready to accept next symbol. |
| rx_sym[1:0] | output | 2-bit RX symbol (0..3) towards controller/PHY MAC side. |
| rx_valid | output | RX symbol valid. |
| rx_ready | input | Handshake from upper layer for RX path. |
| status_err_cnt | output | Optional debug port: accumulated symbol error count in loopback test. |

4. Internal Architecture (Block Diagram View)

At the RTL level, ilc3_ipcore_top is decomposed into three main blocks:
  1) ilc3_tx_core: maps 2-bit data symbols to amplitude levels, applies guard-phase coding, and outputs the quantized amp_tx bus.
  2) ilc3_channel_model: simple in-Rx-domain channel model used only for simulation/verification (gain, offset, optional noise presets). In real silicon this block is replaced by the actual analog channel.
  3) ilc3_rx_core: receives quantized amplitude samples (amp_rx), performs thresholding / detection and guard-phase-aware decoding back to 2-bit symbols.

5. RTL Channel Model & Noise Presets

For IPCore functional and robustness verification, ilc3_channel_model.v implements a simple integer-domain channel with selectable noise presets. The model is not intended to be a physically accurate SNR emulator; instead it provides a convenient way to stress the TX/RX logic around the decision boundaries.

Presets (parameter ADD_NOISE):

0 – clean: no noise, used for pure functional bring-up.

1 – lvl1: strong stress, every symbol gets ±NOISE_LSB; with NOISE_LSB=1, BER ≈ 0.15–0.25 for N_SYM=10k.

2 – lvl2: even stronger, every symbol gets ±2·NOISE_LSB; used only as an extreme corner stress case.

3 – lvl_real: rare ±NOISE_LSB kicks (≈1/64 probability); for N_SYM=100k, measured BER ≈ 1.3e-2. This serves as a qualitative RTL check that the decision boundaries behave as expected around the pre-FEC BER ≈ 1e-2 region, consistent with the Python link simulations.

6. Verification Status (RTL Loopback)

The following loopback tests have been performed on ilc3_ipcore_top using tb_ilc3_ipcore_loopback.v:

clean channel (ADD_NOISE=0): N_SYM = 1k, 10k, 100k → err_cnt = 0 (PASS).

lvl1 stress (ADD_NOISE=1): N_SYM = 10k → err_cnt ≈ 1500 (BER ≈ 1.5e-1).

lvl2 stress (ADD_NOISE=2): N_SYM = 10k → err_cnt ≈ 2000 (BER ≈ 2.0e-1).

lvl_real (ADD_NOISE=3): N_SYM = 100k → err_cnt ≈ 1362 (BER ≈ 1.36e-2).

These results confirm that: (1) the clean path is logically correct and stable over long sequences, and (2) the RX detection logic reacts to small-amplitude perturbations in a way that is qualitatively consistent with the high-SNR Python simulations.

7. Next Steps

1) Freeze this v0.1 architecture as the reference for patent / whitepaper system diagrams.
2) Add more detailed timing diagrams (TX/RX handshake, pipeline latency) and mapping tables (symbol → amplitude → phase).
3) Optionally extend the FEC model to stronger codes and map the resulting SNR improvements on top of the existing ~3 dB gain.
4) Prepare ESP/FPGA testbenches that replace ilc3_channel_model.v with actual SERDES/ADC frontend models.