ILC3_0c_guard-phase IPCore v0.1 – RTL Architecture & Link Spec (Draft)

1. Overview

Purpose. This document describes the first RTL implementation of the ILC3_0c_guard-phase (ILC3_0c_gp) IPCore. The core is designed as a drop-in alternative to PAM4-based PHYs, providing approximately 3 dB SNR gain over PAM4 at the same pre-/post-FEC BER, based on system-level simulations (Q10–Q20).

Scope. The current v0.1 focuses on a clean loopback-capable RTL, with a simple channel model and noise injection, suitable for IP hand-off, verification environments, and future FPGA/ESP prototypes.

2. Top-Level Block and Dataflow

The IPCore is organized around a single top-level module:
    ilc3_ipcore_top
which connects an upper-layer symbol interface to a low-level amplitude (PHY) interface. The typical integration is:
    [Link Layer / FEC] ⇄ [ILC3_IPCore] ⇄ [AFE / DAC / Channel / ADC]

Main blocks inside ilc3_ipcore_top:

TX path: maps 2‑bit symbols (0,1,2,3) to ILC3_0c_gp amplitude levels and drives the serialized AMP bus.

RX path: receives quantized amplitudes from the channel/AFE, performs ILC3_0c_gp decision, and reconstructs 2‑bit symbols.

Handshake logic: simple valid/ready handshake on both symbol and amplitude interfaces.

Optional training/loopback hooks: support for deterministic patterns and internal loopback in the testbench.

3. External Interfaces

3.1 Symbol-side User Interface

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| tx_sym_in | in | 2 | Input symbol from upper layer (00,01,10,11). |
| tx_sym_valid | in | 1 | Asserted by upper layer when tx_sym_in is valid. |
| tx_sym_ready | out | 1 | Asserted by IPCore when it can accept a new tx_sym_in. |
| rx_sym_out | out | 2 | Decoded symbol delivered to upper layer. |
| rx_sym_valid | out | 1 | Asserted when rx_sym_out is valid. |
| rx_sym_ready | in | 1 | Upper layer is ready to consume rx_sym_out (usually tied high). |

Handshake rule: a symbol transfer occurs on a rising clock edge when valid=1 and ready=1. The testbench uses a simple 0,1,2,3,0,1,2,3,… pattern to stress the datapath.

3.2 PHY Amplitude Interface

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| tx_amp_out | out | AMP_WIDTH | Signed fixed-point amplitude toward the DAC / channel. |
| tx_amp_valid | out | 1 | Asserted when tx_amp_out is valid. |
| tx_amp_ready | in | 1 | Downstream (channel/AFE) ready to accept amplitude sample. |
| rx_amp_in | in | AMP_WIDTH | Signed fixed-point amplitude from the channel / ADC. |
| rx_amp_valid | in | 1 | Asserted when rx_amp_in is valid. |
| rx_amp_ready | out | 1 | IPCore ready to accept rx_amp_in. |

AMP_WIDTH is currently 4 bits in the reference RTL, with nominal decision levels chosen to match the Python ILC3_0c_gp simulations (e.g., ±1, ±3 LSB mapping).

4. Internal Architecture (TX/RX Cores)

4.1 TX Core – ilc3_tx_core

Key functions:

Accepts 2‑bit symbols under valid/ready handshake.

Maps 2‑bit symbols to signed amplitude levels consistent with the ILC3_0c_gp constellation.

Provides continuous amplitude stream to the channel (symbol-synchronous, one sample per symbol).

Includes internal buffering to decouple symbol and amplitude handshakes.

4.2 RX Core – ilc3_rx_core

Key functions:

Receives signed amplitudes from the channel/ADC under valid/ready handshake.

Applies the ILC3_0c_guard-phase decision rule (guard-phase based region slicing) to recover the 3-level symbol.

Maps 3-level decisions back to 2‑bit symbols for the upper layer.

Implements simple pipeline stages for timing closure (no DFE required; only feed-forward behavior assumed).

Guard-phase principle: by monitoring transitions and effective ‘guard’ regions between amplitude levels, the receiver can avoid ambiguous decision regions that plague PAM4, leading to approximately 3 dB SNR gain at the same pre-/post-FEC BER when combined with a suitable FFE and block FEC.

5. Channel Model & Noise Presets (Simulation Only)

A reference channel model (ilc3_channel_model) is provided for RTL-level loopback tests. In product integration, this block is replaced by a real AFE + channel. The model supports optional noise injection:

ADD_NOISE = 0: clean loopback (baseline functional test).

ADD_NOISE = 1: strong noise (stress mode). Example result: N_SYM=10k, err_cnt≈2.5k → BER≈2.5e-1.

Future preset (ADD_NOISE = 2): to be tuned as a ‘weak noise’ mode targeting BER≈1e-2 at the RTL level.

These presets are qualitative. Quantitative link-budget and FEC performance are taken from the Python-based ILC3_PData simulations (Q8–Q20), which model realistic 5-tap ISI channels with FFE and block FEC.

6. Link Budget Summary vs PAM4 (Reference)

System-level simulations (Q18–Q20) show that, for 5‑tap ISI channels (a/b/c) with FFE and a (256, 228, t=15) block FEC, ILC3_0c_gp achieves approximately 3 dB SNR gain over PAM4 at both pre-FEC and post-FEC targets:

Pre-FEC BER ≈ 1e-3: ΔSNR ≈ 2.7–2.9 dB in favor of ILC3_0c_gp.

Post-FEC BER targets 1e-6 ~ 1e-12: required SNR for ILC3_0c_gp remains ≈3 dB lower than PAM4 across channels a/b/c.

These results justify treating ILC3_0c_gp as a PAM4-class alternative with ~3 dB link-margin improvement, without requiring DFE on the RX side.

7. Verification Environment

Reference testbenches included in the RTL package:

tb_ilc3_loopback.v – direct core-to-core loopback (no channel), functional sanity check.

tb_ilc3_loopback_noise.v – same as above, with configurable noise in the channel model.

tb_ilc3_ipcore_loopback.v – full IPCore top + channel model loopback, with symbol-side FIFO scoreboard.

The IPCore loopback testbench uses a deterministic pattern (0,1,2,3,…) and reports N_SYM and err_cnt. For example, N_SYM=1024, err_cnt=0 → PASS. By increasing N_SYM (e.g., to 10k) and enabling noise, a rough BER estimate can be obtained at the RTL level.

8. Future Work

Add explicit configuration registers (e.g., to control training mode, noise presets, debug counters).

Connect to a real FEC block and run end-to-end RTL simulations using the same parameters as Q10–Q20.

Prepare FPGA/ESP implementation with on-board DAC/ADC loopback to validate the ~3 dB gain in hardware.

Extend the architecture towards CoPBit-style multi-level/phase processing in a future IPCore family.