# ILC3_0c_guard-phase Algorithm & Link Specification v1.0

Date: 2025-12-01

This document summarizes the algorithm-level behavior and link specification of the ILC3_0c_guard-phase (ILC3_0c_gp) modulation, compared against PAM4, based on Q18–Q20 simulation results with FFE-only equalization and a (256,228,t=15) block FEC.

# 1. Overview

Objective. Provide a link-level specification that can be used as a baseline for IPCore implementation and system-level link budget, showing SNR/BER behavior of ILC3_0c_gp vs. PAM4 under identical channel, equalization, and FEC conditions.

Key result: across three 5-tap ISI channels (a: mild, b: strong, c: medium), ILC3_0c_gp consistently achieves approximately 3 dB SNR gain over PAM4 at the same target BER, both in pre-FEC (e.g., BER≈1e-3) and post-FEC (e.g., BER≈1e-12) regions.

# 2. Modulation and Channel Model

Baseline modulation: 4-level PAM (PAM4) with symbols mapped to levels {-3, -1, +1, +3} (relative scaling).

Proposed modulation: ILC3_0c_guard-phase (ILC3_0c_gp), using 3-level core symbols with guard-phase processing, effectively replacing PAM4 under the same symbol rate and equalization.

Channel models (ISI, 5 taps, normalized):

• Channel a (mild ISI): h = [0.10, 0.40, 1.00, 0.40, 0.10]
• Channel b (strong ISI): h = [0.05, 0.50, 1.00, 0.50, 0.05]
• Channel c (medium ISI): h = [0.075, 0.45, 1.00, 0.45, 0.075]

# 3. Simulation and FEC Setup

- Number of symbols per SNR point: N_sym = 200,000
- SNR sweep: 4, 6, 8, 10, 12, 14, 16, 18, 20 dB (high-SNR extension for Q18)
- Equalization: linear FFE-only, length=11, ridge=0.001, same filter used for PAM4 and ILC3_0c_gp per channel
- Front-end, timing, and phase processing aligned between PAM4 and ILC3_0c_gp for fair comparison.

Block FEC model (used in Q16, Q17, Q20):
- Code length n = 256, information bits k = 228, correction capability t = 15
- Code rate R = 0.8906
- Post-FEC BER approximation: p_post ≈ 0.5 · P_frame_error.

# 4. Pre-FEC SNR Requirements (from Q18/Q19)

Table 1 summarizes the required SNR (in dB) to reach target pre-FEC BERs of 1e-2, 5e-3, and 1e-3 for each channel and scheme.

| Target pre-FEC BER | Channel | Scheme | Required SNR [dB] |
| --- | --- | --- | --- |
| 1e-02 | a | ILC3_0c_gp | 11.934 |
| 1e-02 | a | PAM4 | 14.751 |
| 1e-02 | b | ILC3_0c_gp | 12.164 |
| 1e-02 | b | PAM4 | 15.108 |
| 1e-02 | c | ILC3_0c_gp | 11.985 |
| 1e-02 | c | PAM4 | 14.803 |
| 5e-03 | a | ILC3_0c_gp | 12.678 |
| 5e-03 | a | PAM4 | 15.597 |
| 5e-03 | b | ILC3_0c_gp | 12.907 |
| 5e-03 | b | PAM4 | 16.067 |
| 5e-03 | c | ILC3_0c_gp | 12.729 |
| 5e-03 | c | PAM4 | 15.636 |
| 1e-03 | a | ILC3_0c_gp | 14.247 |
| 1e-03 | a | PAM4 | 16.938 |
| 1e-03 | b | ILC3_0c_gp | 14.442 |
| 1e-03 | b | PAM4 | 17.695 |
| 1e-03 | c | ILC3_0c_gp | 14.252 |
| 1e-03 | c | PAM4 | 16.985 |

Pre-FEC SNR gain summary (PAM4 − ILC3_0c_gp):

Pre-FEC BER=1e-2: avg ΔSNR ≈ 2.860 dB (min=2.817, max=2.944)

Pre-FEC BER=1e-3: avg ΔSNR ≈ 2.892 dB (min=2.691, max=3.253)

# 5. Post-FEC SNR Requirements (from Q20)

Table 2 summarizes the required SNR (in dB) to reach target post-FEC BERs of 1e-6, 1e-9, and 1e-12. These values are obtained by passing the Q18 pre-FEC SNR–BER curves through the (256,228,t=15) FEC model.

| Target post-FEC BER | Channel | Scheme | Required SNR [dB] |
| --- | --- | --- | --- |
| 1e-06 | a | ILC3_0c_gp | 11.222 |
| 1e-06 | a | PAM4 | 14.242 |
| 1e-06 | b | ILC3_0c_gp | 11.508 |
| 1e-06 | b | PAM4 | 14.485 |
| 1e-06 | c | ILC3_0c_gp | 11.266 |
| 1e-06 | c | PAM4 | 14.287 |
| 1e-09 | a | ILC3_0c_gp | 12.000 |
| 1e-09 | a | PAM4 | 14.867 |
| 1e-09 | b | ILC3_0c_gp | 12.000 |
| 1e-09 | b | PAM4 | 15.221 |
| 1e-09 | c | ILC3_0c_gp | 12.000 |
| 1e-09 | c | PAM4 | 14.891 |
| 1e-12 | a | ILC3_0c_gp | 12.000 |
| 1e-12 | a | PAM4 | 15.491 |
| 1e-12 | b | ILC3_0c_gp | 12.000 |
| 1e-12 | b | PAM4 | 15.956 |
| 1e-12 | c | ILC3_0c_gp | 12.000 |
| 1e-12 | c | PAM4 | 15.494 |

Post-FEC SNR gain summary (PAM4 − ILC3_0c_gp):

Post-FEC BER=1e-6: avg ΔSNR ≈ 3.006 dB (min=2.977, max=3.021)

Post-FEC BER=1e-12: avg ΔSNR ≈ 3.647 dB (min=3.491, max=3.956)

At post-FEC BER ≈ 1e-12, ILC3_0c_gp requires approximately 12 dB SNR across channels a/b/c, while PAM4 requires roughly 15.5–16 dB. This corresponds to a post-FEC SNR gain of approximately 3.5 dB, consistent with the pre-FEC ~3 dB gain observed around BER ≈ 1e-3.

# 6. Link Budget Interpretation

- The ~3 dB SNR gain can be interpreted as:
  • For a fixed target BER (e.g., post-FEC 1e-12), ILC3_0c_gp can operate at roughly half the transmit power compared to PAM4.
  • For a fixed transmit power, ILC3_0c_gp provides approximately 3 dB additional link margin.
- This holds across different ISI severities (channels a/b/c), indicating that the guard-phase gain is robust to channel shape when FFE-only equalization is used.

# 7. IPCore Design Baseline

The RTL ILC3 IPCore is intended to implement the same algorithmic path used in the Q18–Q20 simulations:
- Same symbol mapping and guard-phase processing as in ILC3_0c_gp baseband model.
- Same FFE-only equalization structure at the receiver (or a hardware-efficient approximation).
- Identical channel assumptions (5-tap ISI, AWGN with equivalent SNR definition).
The tables above therefore serve directly as the initial link budget and product-level specification for the IPCore-based PHY.